### B.4 — Extended Hamilton’s Principle for Hybrid (Discrete–Continuous) Dynamical Systems

Real aerospace structures are almost never purely rigid or purely flexible.
Examples:

- a launch vehicle that bends and vibrates during ascent,

- a spacecraft bus with long solar arrays,

- an aircraft wing with distributed aerodynamic loading.

To describe such systems we need two kinds of coordinates:

- Discrete (lumped) coordinates qi(t)qi​(t)

  - Finite set of generalized coordinates.

  - Capture rigid-body motion, hinge angles, actuator strokes, etc.

- Distributed (field) coordinates wj(x,t)wj​(x,t)

  - Functions of space and time.

  - Capture deformation along a beam, torsion of a panel, vibration of an appendage, etc., over a domain x∈Ωx∈Ω.

Standard Lagrange equations handle only the finite-dimensional part. Classical field theory treats only the PDE part.
Extended Hamilton’s principle gives a single variational framework that handles both together and automatically generates:

- ODEs for qi(t)qi​(t),

- PDEs for wj(x,t)wj​(x,t),

- and the associated boundary/interface conditions.

#### 1. Hybrid Lagrangian: rigid, flexible, and interface terms

For a hybrid system it is convenient to split the total Lagrangian into three pieces

L=LD(q,q˙,t)+LB(q,w,w˙,t)+∫ΩL^(q,q˙,w,w˙,w′,w˙′,x,t) dΩ.L=LD​(q,q˙​,t)+LB​(q,w,w˙,t)+∫Ω​L^(q,q˙​,w,w˙,w′,w˙′,x,t)dΩ.

- LDLD​ – Discrete (rigid-body) part
Kinetic and potential energy of rigid masses, lumped actuators, etc.

- LBLB​ – Boundary/interface part
Energy terms associated with joints or attachment points where the rigid and flexible subsystems meet (e.g. root bending moment times root deflection).

- L^L^ – Distributed Lagrangian density
Integrated over the elastic domain ΩΩ. It typically includes

  - distributed kinetic energy 12ρA w˙221​ρAw˙2,

  - strain energy depending on spatial derivatives w′,w′′,…w′,w′′,…,

  - distributed external loads (aerodynamic pressure, gravity gradient, etc.).

This decomposition keeps the bookkeeping clean:

- rigid variables (q,q˙)(q,q˙​),

- flexible field (w,w˙)(w,w˙),

- and their coupling at the interface.

#### 2. Extended Hamilton’s principle with nonconservative effects

Allow a (possibly non-zero) virtual work of nonconservative forces δWncδWnc​.
The extended principle states that the actual motion satisfies

δ∫t0tf(L−Wnc) dt=0,δ∫t0​tf​​(L−Wnc​)dt=0,

for all admissible variations

- δqi(t)δqi​(t) of discrete coordinates,

- δwj(x,t)δwj​(x,t) of distributed fields,

- and compatible boundary variations δwj(0,t)δwj​(0,t), δwj(ℓ,t)δwj​(ℓ,t) (unless those are fixed).

Here WncWnc​ collects generalized nonconservative work terms, for example:

- rigid generalized forces Qi(t) δqiQi​(t)δqi​,

- distributed loads f^(x,t) δw(x,t)f^​(x,t)δw(x,t),

- boundary forces/moments acting at the interface.

Because w(x,t)w(x,t) depends on both xx and tt, the variational calculus requires integration by parts in time and in space. This is where the hybrid nature shows up.

#### 3. Resulting equations for the discrete coordinates

Collect all terms that multiply δqi(t)δqi​(t). After integrating by parts in time and using that δqi(t0)=δqi(tf)=0δqi​(t0​)=δqi​(tf​)=0, we obtain generalized Lagrange equations of the form

ddt(∂L∂q˙i)−∂L∂qi=Qi(T),dtd​(∂q˙​i​∂L​)−∂qi​∂L​=Qi(T)​,

where Qi(T)Qi(T)​ is the total generalized nonconservative force associated with qiqi​. It can contain contributions from

- lumped actuators and damping,

- reaction forces transmitted by the flexible structure through LBLB​ and L^L^.

So, even though the equation looks like the classical finite-dimensional Lagrange equation, every term now includes the effects of the distributed subsystem. The rigid-body motion is “aware” of the elastic deformation.

#### 4. Resulting equations for the distributed coordinates

Next, collect terms multiplying δw(x,t)δw(x,t) in the interior of the domain.
After integrating by parts in both tt and xx, the interior variation yields a PDE of Euler–Lagrange type

ddt(∂L^∂w˙)−∂L^∂w+∂∂x(∂L^∂w′)−∂2∂x2(∂L^∂w′′)=f^(T)(x,t),dtd​(∂w˙∂L^​)−∂w∂L^​+∂x∂​(∂w′∂L^​)−∂x2∂2​(∂w′′∂L^​)=f^​(T)(x,t),

where f^(T)f^​(T) represents the net distributed nonconservative loading per unit length (or area, volume, etc.).

Interpretation:

- the time-derivative term comes from distributed kinetic energy,

- the spatial-derivative terms come from bending, torsion, or axial strain energies,

- the right-hand side collects external loads and damping.

For an Euler–Bernoulli beam, for instance, choosing the usual kinetic and bending energies in L^L^ recovers the familiar fourth-order beam equation.

#### 5. Boundary and interface conditions (natural vs essential)

The remaining terms in the variation involve

δw(0,t),δw(ℓ,t),δw′(0,t),δw′(ℓ,t),δw(0,t),δw(ℓ,t),δw′(0,t),δw′(ℓ,t),

and the coupling with the boundary Lagrangian LBLB​. These terms must vanish independently of the interior variations, leading to the boundary/interface conditions.

In compact form the boundary part has the structure

[(∂L^∂w′−∂∂x∂L^∂w′′)δw+(∂L^∂w′′)δw′+(boundary forces and moments)]x=0x=ℓ=0.[(∂w′∂L^​−∂x∂​∂w′′∂L^​)δw+(∂w′′∂L^​)δw′+(boundary forces and moments)]x=0x=ℓ​=0.

Depending on how the variations are restricted, we get two families of conditions:

- Essential (geometric) boundary conditions

If the displacement or slope is prescribed, the corresponding variation is zero. Examples:

  - clamped end at x=0x=0: w(0,t)=0w(0,t)=0 and w′(0,t)=0w′(0,t)=0,

  - pinned end: w(0,t)=0w(0,t)=0 but bending moment free.

These are imposed directly on the admissible variations.

- Natural (dynamic) boundary conditions

If a variation is not constrained at a boundary, its coefficient must vanish. This produces physical conditions such as

  - zero bending moment or zero shear force at a free end,

  - equality of interface shear/moment between a rigid hub and a flexible appendage,

  - applied endpoint forces and moments.

The important point: Hamilton’s principle delivers the boundary conditions automatically, without guessing them first.

#### 6. Energy viewpoint and link to Hamiltonian dynamics

For a conservative hybrid system (no explicit time dependence in LL, no nonconservative forces), the extended Hamilton principle is closely tied to an energy integral.

- One can define generalized momenta for both parts:

  - lumped momenta pi=∂L/∂q˙ipi​=∂L/∂q˙​i​,

  - momentum density for the field π(x,t)=∂L^/∂w˙(x,t)π(x,t)=∂L^/∂w˙(x,t).

- A hybrid Hamiltonian can then be formed,

H=∑ipiq˙i+∫Ωπ w˙ dΩ−L,H=i∑​pi​q˙​i​+∫Ω​πw˙dΩ−L,

mirroring the finite-dimensional definition from Chapter 7.

Under the usual “natural system” assumptions (kinetic energy quadratic in velocities, potential energy depending only on configuration), HH reduces to the total mechanical energy

H=T+V,H=T+V,

and is conserved when nonconservative forces vanish.

# B.4 — Extended Hamilton’s Principle for Hybrid (Discrete–Continuous) Dynamical Systems

(Fully rewritten, expanded, and non-plagiarized)

Real aerospace structures are rarely purely rigid or purely elastic. A launch vehicle bends during ascent, a satellite’s solar panels vibrate, and aircraft wings undergo distributed aerodynamic loading. Such systems contain two distinct types of coordinates:

### 1. Discrete Coordinates qi(t)qi​(t)

These represent rigid-body or lumped motions such as translation, rotation, or hinge deflections.

### 2. Distributed Coordinates wj(x,t)wj​(x,t)

These represent deformation fields defined over a spatial domain—e.g., bending of a beam along its span xx, torsion of a panel, or vibration of a flexible appendage.

Classical Lagrange equations only handle systems described by a finite number of coordinates.
Classical field theory handles PDE systems but cannot easily describe rigid–flexible coupling.
Hybrid systems require a unified variational principle.

Extended Hamilton’s principle provides this unification.

# 1. Hybrid Lagrangian Structure

The total Lagrangian of a hybrid system naturally splits into three components:

L=LD(q,q˙)+LB(q,w,w˙)+∫ΩL^(q,q˙,w,w˙,w′,w˙′,x) dΩL=LD​(q,q˙​)+LB​(q,w,w˙)+∫Ω​L^(q,q˙​,w,w˙,w′,w˙′,x)dΩ

where:

### (a) LDLD​ — Discrete (rigid-body) contribution

Contains kinetic/potential energy of rigid masses or lumped joints.

### (b) LBLB​ — Boundary coupling contribution

Accounts for the interaction between rigid subsystems and the flexible domain boundaries—for example, forces and moments transmitted at a joint or attachment point.

### (c) L^L^ — Distributed energy density

Integrated over the elastic domain, representing:

- distributed kinetic energy,

- strain energy (involving w′w′ and possibly w′′w′′),

- distributed external forces.

This decomposition allows rigid-body and elastic degrees of freedom to be treated on equal footing.

# 2. Statement of Extended Hamilton’s Principle

For a hybrid system subjected to possible nonconservative effects, the principle states:

δ∫t0tf(L−Wnc) dt=0δ∫t0​tf​​(L−Wnc​)dt=0

Variations are taken with respect to:

- discrete variations δqi(t)δqi​(t),

- distributed variations δwj(x,t)δwj​(x,t),

- boundary variations δwj(0,t),δwj(l,t)δwj​(0,t),δwj​(l,t) unless fixed.

Because wjwj​ depends on both xx and tt, the variation must include integration-by-parts in both variables.
This produces ordinary differential equations, partial differential equations, and natural boundary conditionssimultaneously—something the classical finite-dimensional Lagrange method cannot do.

# 3. Governing Equations Produced by the Variational Process

After performing the variational steps—integrating time-terms by parts, then spatial-terms by parts—the variation separates into interior and boundary contributions.

## A. Discrete Coordinate Equations (ODEs)

These follow from terms multiplying δqi(t)δqi​(t):

ddt(∂L∂q˙i)−∂L∂qi=QiT(6.80, discrete part)dtd​(∂q˙​i​∂L​)−∂qi​∂L​=QiT​(6.80, discrete part)

They resemble the classical Lagrange equations, but here each term includes back-reaction from flexible deformations through the coupling terms LBLB​ and L^L^.

Thus, the rigid-body dynamics “feels” the elastic motion.

## B. Distributed Field Equations (PDEs)

These follow from interior domain variations δw(x,t)δw(x,t):

ddt(∂L^∂w˙)−∂L^∂w+∂∂x(∂L^∂w′)−∂2∂x2(∂L^∂w′′)=f^T(6.81)dtd​(∂w˙∂L^​)−∂w∂L^​+∂x∂​(∂w′∂L^​)−∂x2∂2​(∂w′′∂L^​)=f^​T(6.81)

These are the Euler–Lagrange PDEs for the elastic domain.
They express balance of:

- inertial effects (from ∂L^/∂w˙∂L^/∂w˙),

- elastic restoring forces (from spatial derivatives),

- distributed nonconservative effects f^Tf^​T.

This PDE describes the vibration or deformation of flexible components.

# 4. Natural Boundary Conditions from Hamilton’s Principle

Terms multiplying δw(0,t)δw(0,t), δw(l,t)δw(l,t), δw′(0,t)δw′(0,t), δw′(l,t)δw′(l,t) yield boundary/interface conditions.

From the extended variation you obtain the boundary expressions:

{∂L^∂w′−∂∂x(∂L^∂w′′)}x=0x=lδw+{∂L^∂w′′}x=0x=lδw′+boundary forces=0(6.82–6.83){∂w′∂L^​−∂x∂​(∂w′′∂L^​)}x=0x=l​δw+{∂w′′∂L^​}x=0x=l​δw′+boundary forces=0(6.82–6.83)

Depending on whether the variations vanish or not, you obtain:

### • Essential (geometric) boundary conditions

If δw=0δw=0 or δw′=0δw′=0, then displacement or slope is prescribed.

### • Natural (dynamic) boundary conditions

If variations are free, the multipliers must vanish, yielding physical conditions such as:

- zero bending moment,

- zero shear force,

- consistent rigid–flexible coupling conditions,

- applied forces/moments at boundaries.

Unlike the classical method, Hamilton’s principle automatically generates boundary conditions without requiring prior physical assumptions.

# 5. Interpretation of the Full Hybrid System

Equations (6.80) and (6.81) provide the governing ODE–PDE dynamics.
Equations (6.82) and (6.83) provide the associated boundary/interface conditions.

Together, these form a coupled system describing:

- rigid-body motion (finite-dimensional),

- elastic deformation (infinite-dimensional),

- their mutual coupling through shared energy terms.

This unified framework is essential for modeling:

- launch vehicles with bending and torsion modes,

- spacecraft appendages (booms, antennas, solar arrays),

- flexible aircraft wings and fuselage dynamics,

- soft robotics and slender manipulators,

- any structure where rigid and elastic motions interact.

The extended Hamilton formulation is the foundation for modern flexible multibody dynamics, allowing consistent derivation of ODE–PDE systems for realistic aerospace structures.

# B.5 — Hamiltonian Dynamics & Canonical Transformations

A unified, symplectic framework for advanced analytical dynamics

Hamiltonian dynamics provides an alternative to the Lagrangian formulation.
Where Lagrange’s equations describe motion using generalized coordinates qiqi​ and velocities q˙iq˙​i​, the Hamiltonian picture uses coordinates qiqi​ and their conjugate momenta

pi=∂L∂q˙i.pi​=∂q˙​i​∂L​.

The resulting framework reveals powerful geometric properties of dynamics—especially symplectic structure and canonical transformations—which are central to orbital mechanics, spacecraft attitude dynamics, and modern numerical simulation.

## 1. Constructing the Hamiltonian

Given a Lagrangian L(q,q˙,t)L(q,q˙​,t), define the conjugate momenta

pi=∂L∂q˙i,i=1,…,n.pi​=∂q˙​i​∂L​,i=1,…,n.

Assuming the relationship between q˙q˙​ and pp can be inverted, we define the Hamiltonian

H(q,p,t)=∑i=1npiq˙i−L(q,q˙,t),H(q,p,t)=i=1∑n​pi​q˙​i​−L(q,q˙​,t),

expressed entirely in terms of q,p,tq,p,t.

For natural systems (kinetic energy quadratic in q˙q˙​, potential depending only on qq),

H=T+V,H=T+V,

so the Hamiltonian equals the total energy.

## 2. Hamilton’s Canonical Equations

Taking variations of the action in Hamiltonian form (or transforming Lagrange’s equations) produces the canonical equations of motion:

q˙i=∂H∂pi,p˙i=−∂H∂qi+Qnc,i,(7.66)q˙​i​=∂pi​∂H​,p˙​i​=−∂qi​∂H​+Qnc,i​,(7.66)

where Qnc,iQnc,i​ are generalized nonconservative forces.

In vector–matrix form:

x˙=[J]−1∂H∂x+[0Qnc],x=[qp],(7.68)x˙=[J]−1∂x∂H​+[0Qnc​​],x=[qp​],(7.68)

with the symplectic matrix

[J]=[0−II   0].(7.69)[J]=[0I​−I0​].(7.69)

The matrix [J][J] encodes the geometric structure of phase space.

## 3. Poisson Brackets — Compact Form of Hamilton’s Equations

For any smooth function
F(q,p,t)F(q,p,t), its time derivative is

dFdt=(F,H)+∂F∂t+[∂F∂p]TQnc,(7.74)dtdF​=(F,H)+∂t∂F​+[∂p∂F​]TQnc​,(7.74)

where the Poisson bracket is defined as

(F,G)=∑i=1n(∂F∂qi∂G∂pi−∂F∂pi∂G∂qi).(7.75)(F,G)=i=1∑n​(∂qi​∂F​∂pi​∂G​−∂pi​∂F​∂qi​∂G​).(7.75)

Hamilton’s equations themselves are equivalent to:

q˙i=(qi,H),p˙i=(pi,H)+Qnc,i.q˙​i​=(qi​,H),p˙​i​=(pi​,H)+Qnc,i​.

A function FF is conserved if

∂F∂t=0,(F,H)=0,∂F∂pQnc=0.∂t∂F​=0,(F,H)=0,∂p∂F​Qnc​=0.

## 4. Canonical Transformations

A transformation

(q,p)⟶(Q,P)(q,p)⟶(Q,P)

is called canonical if the new variables satisfy Hamilton’s equations with a (possibly) new Hamiltonian H∗(Q,P,t)H∗(Q,P,t).
This requires that the transformation preserve the symplectic structure of phase space.

### 4.1 Symplectic (Jacobian) Criterion

Let the transformation be smooth and invertible:

Q=f(q,p,t),P=g(q,p,t).Q=f(q,p,t),P=g(q,p,t).

Define the Jacobian matrix:

[M]=[∂Q∂q∂Q∂p∂P∂q∂P∂p].(7.112–7.115)[M]=[∂q∂Q​∂q∂P​​∂p∂Q​∂p∂P​​].(7.112–7.115)

The transformation is canonical iff

[M][J][M]T=[J].(7.119)[M][J][M]T=[J].(7.119)

This is the symplectic condition.

- It ensures preservation of phase-space volume (Liouville’s theorem).

- It preserves Poisson brackets:
(Qi,Pj)=δij(Qi​,Pj​)=δij​, (Qi,Qj)=0(Qi​,Qj​)=0, (Pi,Pj)=0(Pi​,Pj​)=0.

## 5. Perfect Differential Criterion (Powerful Test)

Instead of checking the symplectic matrix equation, one may use the simpler condition:

∑i=1n(qi dpi−pi dqi)−∑i=1n(Qi dPi−Pi dQi)=dW,(7.95)i=1∑n​(qi​dpi​−pi​dqi​)−i=1∑n​(Qi​dPi​−Pi​dQi​)=dW,(7.95)

for some differentiable scalar function WW.

If the left-hand side is an exact differential dWdW,
the transformation is canonical.

This criterion is much easier for hand calculations.

## 6. Generating Functions

Most canonical transformations can be generated from a scalar function WW of mixed old/new variables:

| Type | Generating Function | Relations |
| --- | --- | --- |
| W1(q,Q,t)W1​(q,Q,t) | Depends on old qq, new QQ | p=∂W1∂q, P=−∂W1∂Qp=∂q∂W1​​,P=−∂Q∂W1​​ |
| W2(q,P,t)W2​(q,P,t) | Most common in orbital mechanics | p=∂W2∂q, Q=∂W2∂Pp=∂q∂W2​​,Q=∂P∂W2​​ |
| W3(p,Q,t)W3​(p,Q,t) | Momentum-based | q=−∂W3∂p, P=−∂W3∂Qq=−∂p∂W3​​,P=−∂Q∂W3​​ |
| W4(p,P,t)W4​(p,P,t) | Rare | q=−∂W4∂p, Q=∂W4∂Pq=−∂p∂W4​​,Q=∂P∂W4​​ |

Generating functions are fundamental in:

- orbit element transformations,

- Hamilton–Jacobi theory,

- action–angle variables.

## 7. Example: Orthogonal Canonical Transformation (from your screenshot)

If

q=[A]q∗,p=[A]p∗,(7.105)q=[A]q∗,p=[A]p∗,(7.105)

with an orthogonal matrix AA such that

ATA=I,(7.106)ATA=I,(7.106)

then the transformation is canonical because

qTdp−q′Tdp∗=0(7.107)qTdp−q′Tdp∗=0(7.107)

is identically satisfied ⇒ perfect differential condition.

This often appears in modal transformations (e.g., converting coupled oscillators to normal modes).

## 8. Example: Nonlinear Transformation (Example 7.7)

For the transformation

qj=2ajcos⁡pj∗,pj=2ajsin⁡pj∗,(7.108)qj​=2aj​​cospj∗​,pj​=2aj​​sinpj∗​,(7.108)

the perfect differential condition yields

W=−12∑jajsin⁡(2pj∗).(7.109)W=−21​j∑​aj​sin(2pj∗​).(7.109)

Hence it is canonical, and the new Hamiltonian is obtained by substituting into H(q,p)H(q,p).

## 9. Time as a Canonical Coordinate

If the Hamiltonian explicitly depends on time, we can extend phase space by adding:

qn+1=t,pn+1=−H,(7.80–7.82)qn+1​=t,pn+1​=−H,(7.80–7.82)

which converts a time-dependent Hamiltonian into an autonomous system.
This is extremely useful in orbit perturbation theory.

## 10. Summary (Exam-Ready)

Hamiltonian Dynamics

- Define momenta: pi=∂L/∂q˙ipi​=∂L/∂q˙​i​.

- Hamiltonian: H=∑piq˙i−LH=∑pi​q˙​i​−L.

- Canonical equations:
q˙i=∂H/∂piq˙​i​=∂H/∂pi​,
p˙i=−∂H/∂qi+Qnc,ip˙​i​=−∂H/∂qi​+Qnc,i​.

- Poisson bracket form: F˙=(F,H)+∂F/∂t+FpQncF˙=(F,H)+∂F/∂t+Fp​Qnc​.

Canonical Transformations

- Preserve Hamilton’s equation structure.

- Symplectic condition:
[M][J][M]T=[J][M][J][M]T=[J].

- Perfect differential test: LHS = dWdW.

- Generated by functions W1,W2,W3,W4W1​,W2​,W3​,W4​.

- Allow simplification of dynamics, change of coordinates, normal modes, and orbit element transformations.