## Domain E.5 — STK (Systems Tool Kit) for Mission Analysis

A system-level framework for scenario construction, coverage assessment, trade studies, and model validation

STK sits at the “mission meaning” layer of astrodynamics. Your analytical and MATLAB models tell you where the state is and how it evolves under chosen physics. STK tells you what that state does when you embed it inside a complete operational scenario—Earth geometry, sensors, pointing, access rules, downlinks, and time-ordered events. In Domain E, STK is therefore not a replacement for equations or custom simulation. It is the bridge between dynamicsand mission outcomes.

STK

## 1) Why STK matters in mission analysis

Modern missions don’t fail because Kepler’s laws are wrong. They fail because mission performance is shaped by interactions that are difficult to keep consistent in a purely analytical pipeline:

- Dynamics (orbits, perturbations, relative motion)

- Geometry (Earth shape, occultation, horizon effects, terrain if enabled)

- Sensors (FOV, pointing, slew limits, keep-out zones, duty cycles)

- Operations (tasking, scheduling conflicts, comm windows, constraints)

- Time (sampling, rise/set events, eclipse transitions, local solar time)

STK exists to make those interactions explicit and consistent. In practice, STK plays three roles:

- Scenario integrator: one environment where all objects share the same time span, frames, and Earth model.

- Visual reasoning tool: it makes “why is this happening?” answerable in geometry.

- Validation reference: it exposes hidden assumptions in MATLAB/analytical results by forcing constraints to be modeled explicitly.

STK

## 2) Scenario construction: making the mission question well-posed

### 2.1 What a “scenario” actually is

A scenario is not cosmetic. It is the formal definition of your mission question:

- Start/stop time (what operational window are you evaluating?)

- Earth model and reference frames (sphere vs WGS-84, ECI vs ECEF, etc.)

- Time step / event detection rules (do you sample or detect continuous events?)

- Fidelity choices (terrain, atmosphere, lighting constraints as applicable)

Until these are fixed, “what does this orbit do?” has multiple valid answers.

### Scenario A — “My access changed when I shifted epoch. Why?”

What you observe: You run the same orbit but change the start date; access and coverage metrics shift.
Why it happens: Earth rotates under the orbit, and Sun–Earth–satellite geometry changes with epoch. That alters:

- where ground tracks fall in longitude over the window,

- lighting/eclipses for optical payloads,

- when ground assets rotate into view,

- even apparent “coverage clustering” over days.
What STK gives you: A single consistent temporal context so your results are tied to an operational timeline—not an abstract orbit.

STK

### Scenario B — “One tool shows access; another misses it.”

What you observe: MATLAB sampled every 60 s; STK shows short access windows you missed (or MATLAB shows access that disappears).
Why it happens: Access can be short-lived. Coarse sampling can:

- miss brief passes entirely,

- mis-estimate peak elevation,

- violate minimum-duration constraints.
What STK gives you: Event-level rise/set detection and explicit time-resolution logic, reducing “sampling artifacts.”

STK

### Scenario C — “MATLAB sees the satellite, STK doesn’t.”

What you observe: Your MATLAB line-of-sight says visible; STK says occulted or below mask.
Why it happens: Different geometric assumptions:

- spherical Earth vs WGS-84 ellipsoid,

- ignoring station altitude/horizon depression,

- missing terrain masking or elevation masks.
What STK gives you: A consistent geometry engine so visibility is evaluated using explicit Earth/terrain assumptions.

STK

### Scenario D — “Frame choice changed my access times.”

What you observe: You propagate in ECI, but your ground station and target are Earth-fixed; access shifts mysteriously.
Why it happens: If ECI↔ECEF transforms or Earth rotation are inconsistently applied, access timing and pointing can be wrong.
What STK gives you: Frame-aware geometry: inertial motion + Earth-fixed assets interact correctly by construction.

STK

Core takeaway: Scenario definition doesn’t change the orbit. It changes the meaning of the mission question—and makes it reproducible.

## 3) Object-based modeling: why it matters conceptually

STK represents mission elements as interacting objects—satellites, sensors, ground stations, targets, constellations—each carrying:

- Dynamics (orbit/motion model)

- Geometry (shape, FOV, pointing)

- Operational logic (tasking rules, access constraints)

This mirrors real programs: performance emerges from interactions, not from a single equation set.

### Scenario E — “What is limiting my mission: orbit, sensor, or operations?”

What you observe: Coverage isn’t meeting requirements.
The conceptual problem: Without object separation, it’s easy to blame the orbit when the real bottleneck is pointing, duty cycle, or downlink.
What STK lets you do: Isolate causes by toggling one object capability at a time:

- Change orbit altitude/inclination → see orbit-limited effects

- Change FOV/pointing limits → see sensor-limited effects

- Change duty cycle/schedule → see operations-limited effects
Outcome: You can say what is limiting performance and why—which is the essence of mission analysis.

STK

## 4) Orbit visualization: not cosmetic—diagnostic

STK visualization makes the hidden structure behind metrics visible:

- ground tracks evolving over days (Earth rotation beneath inertial orbit)

- eclipse/lighting geometry

- relative motion between objects

- sensor footprints and when/where they actually cover

### Scenario F — “Why does coverage collapse in one region at certain times?”

What you observe: Average revisit looks fine, but one region has long gaps.
Why it happens: It’s usually a geometry-time intersection: phasing + Earth rotation + constraints.
What STK shows you visually:

- footprints repeatedly missing the region due to phasing,

- passes removed by eclipse/pointing rules,

- ground tracks drifting relative to longitude over the analysis window.
Outcome: The “mysterious metric” becomes explainable behavior, not a spreadsheet surprise.

STK

### Scenario G — “Why are poles great but equator poor?”

Why it happens: Ground tracks converge at high latitudes; low latitudes require deliberate phasing/architecture for persistence.
What STK reveals: Tracks stacking near poles and spreading thin near the equator—instantly clarifying why latitude drives performance.

STK

### Scenario H — “Why does latency spike at certain local times?”

Why it happens: Not dynamics—usually Earth-fixed infrastructure rotating out of view, combined with eclipse/power constraints.
What STK reveals: Where the satellite is relative to ground stations and lighting—so latency spikes become geometry-driven, not mysterious.

STK

## 5) Coverage analysis: turning access into mission performance

Orbit propagation answers: where do I go?
Coverage answers: does the mission work?

STK converts access events into mission-level metrics:

- revisit distributions (not just the mean)

- worst-case gaps (often the real requirement driver)

- dwell-time distributions

- percent area coverage over time

- latency to observation / latency to downlink (depending on setup)

### Scenario I — “We have access, but do we have persistence?”

What you observe: You can “see” the target sometimes, but mission needs continuous awareness.
Why access isn’t enough: A single pass per day may be operationally useless.
What STK provides: Gap statistics and revisit distributions—so persistence is evaluated by maximum blind time, not by “access exists.”

STK

### Scenario J — “Are we revisit-limited or dwell-limited?”

Two designs can share the same mean revisit but differ in dwell time and usefulness.
What STK provides: Separate histograms for revisit and dwell, plus correlation—so you can identify the true bottleneck.

STK

### Scenario K — “Are we meeting requirements everywhere, or only on average?”

What fails in many designs: Global averages look great while priority regions fail badly.
What STK provides: Geographic coverage maps and worst-case regions—so performance is tested where it matters.

STK

## 6) Sensor geometry & access: access is geometry + constraints

Access is not “the orbit is nearby.” It is “a valid measurement is feasible.”

STK access explicitly accounts for:

- Earth occultation and limb geometry

- elevation masks and horizon limits

- sensor FOV shape and boresight

- pointing/slew/keep-out constraints

- (if enabled) terrain/atmosphere and lighting constraints

### Scenario L — “Ground track passes near my station, but no access.”

Why it happens: Ground track proximity is 2-D; visibility is 3-D and mask-limited.
STK’s answer: True line-of-sight + elevation constraints reveal whether Earth curvature/masks block the pass.

STK

### Scenario M — “Space-based sensor misses targets at certain angles.”

Why it happens: Off-nadir pointing limits, FOV geometry, or slew-rate constraints prevent acquisition even when range is small.
STK’s answer: It distinguishes “visible” from “pointable,” which is the real mission condition.

STK

### Scenario N — “Is this an orbit issue, or a geometry/pointing issue?”

How STK resolves it cleanly:

- If you import the same ephemeris and mismatch remains → constraints/geometry dominate.

- If mismatch disappears → propagation/dynamics assumptions dominate.
That single logic step prevents weeks of debugging the wrong thing.

STK

## 7) Constellations & trade studies: performance is not linear with satellite count

Constellations reshape time (revisit, gaps, latency) and redistribute geometry. STK supports rapid creation (Walker patterns, custom phasing, mixed inclinations) and, more importantly, objective comparison via metrics.

### Scenario O — “How many satellites do we actually need?”

Why theory alone struggles: Mean revisit might improve while worst-case gaps remain unacceptable.
What STK provides: Where requirements are satisfied (and where returns diminish), using distributions rather than a single mean.

STK

### Scenario P — “Is my constellation redundant or complementary?”

Why it matters: Pass clustering can inflate “total access time” while leaving worst-case gaps untouched.
What STK reveals: Whether added satellites shrink maximum gaps or merely duplicate already-covered periods.

STK

### Scenario Q — “Should I add planes or add sats per plane?”

Typical behavior:

- more planes → improves global longitudinal spread and reduces global gaps

- more sats/plane → densifies coverage along existing tracks
STK’s role: It quantifies which move actually improves your mission metric (gap, latency, revisit variance), rather than relying on intuition.

STK

### Scenario R — “Two architectures have similar coverage—how do we choose?”

Why “coverage %” can mislead: Similar coverage can hide different time structures: clustered gaps vs evenly spaced access, short frequent dwells vs fewer longer dwells.
What STK provides: side-by-side comparisons of gap percentiles, latency distributions, dwell statistics—so “similar coverage” becomes a meaningful decision.

STK

### Scenario S — “How do I justify one more sensor or station to leadership?”

Leadership needs cause → effect: If we add X, we gain Y.
STK provides defensible numbers:

- worst-case gap reduction

- latency improvement in priority regions

- additional targets maintained with custody

- robustness when one asset fails
That turns a budget request into a measurable performance upgrade.

STK

## 8) Validation with MATLAB/analytical models: STK as “assumption exposure”

Analytical/MATLAB models excel at transparency and custom physics. STK excels at integrated geometry and operational realism. Used together, you gain confidence and understanding.

### 8.1 Why validation is needed

Most “disagreements” come from assumptions you didn’t realize you were making:

- spherical vs ellipsoidal Earth

- missing elevation masks/terrain

- idealized pointing

- ignored eclipse/lighting rules

- coarse sampling steps

- missing tasking conflicts

STK forces those assumptions into explicit model elements—so you can see what changed and why.

STK

### 8.2 Cross-validation as a reasoning method (not a “check”)

The workflow is simple, but the mindset is the key:

- MATLAB predicts: orbit + access + revisit

- STK predicts: orbit + access + revisit

- If mismatch occurs, don’t ask “who is wrong?” Ask: what assumption does the mismatch expose?

#### Scenario T — “Our access windows don’t match. What do we test first?”

A structured diagnostic sequence usually resolves it quickly:

- Time alignment: start/stop, epoch definitions, UTC handling

- Earth model: sphere vs WGS-84, station altitude, terrain

- Masks: minimum elevation, keep-out zones

- Sensor modeling: FOV shape, boresight, pointing limits

- Sampling vs events: discrete sampling artifacts vs event detection

Each mismatch is a symptom with a cause. When you interpret it that way, validation becomes a tool for discovering what dominates mission performance (geometry, operations, architecture), not just verifying code.

STK

One-line takeaway: STK cross-validation turns disagreement into understanding by revealing which assumptions actually control your mission.

## 9) Key takeaways

- STK is a mission analysis + validation environment, not a black-box propagator.

- Scenario framing makes mission questions well-posed and reproducible.

- Coverage and access are fundamentally geometry + time + constraints problems.

- Sensor modeling connects orbit mechanics to observability, not just visibility.

- Constellation design is a trade study—you choose architectures by metrics (gaps, latency, dwell), not by satellite count.

- Analytical/MATLAB + STK together is strongest: equations for insight, STK for operational truth checking.

STK