Particle Position Description

The cartesian coordinate system:

Needs origin A to specify its position in space

And orientation of coordinates System

the origin and the three unit direction

vectors, is often referred to as a reference frame. Vectors with components

taken in different coordinate systems are said to be written in

different reference frames.

Let particle is in 3 dimensional space with reference sustem R and distance from Origin O is d. The three unit vectors r1, r2,

and r3 are used to define the orientation of R.

d=xr1+yr2+zr3

Can be expressed in column matrix of 3 x 1

While doing  vectot operation of different reference systems, consider to project components to one system, then do operation.

Cylindrical  coordinate system: describes particle is in rotational motion or dominant force is radial . if particle is rotating about a particular axis , it is fre to move parallel to that axis. Other 2 oriented unit vectors varying at angle in plane perpendicular to former.

In column matrix one entry is always 0.

ed=cosθ r1+sinθr2

eθ=-sinθ r1+cosθr2

Spherical coordinate system  describes a particle motion on the surface of a sphere or a particle orbiting a body. The orientation is described by 2 angles θ and ∅.

gd=cos∅ cosθ r1+cos∅ sinθr2+sin∅ r3

gθ=-sinθ r1+cosθr2

g∅=-sin∅cosθ r1-sin∅sinθr2+cos∅r3

Angular velocity vector

It describes at what rate the radius vector locating the particle is orbiting the origin. If body is rotating about fixed axis with respect to the body and space by small  angle ∆θ. The angular velocity vector ω=θ

Particle Kinematics with Moving Frames

If particle is in 3D space and at distance d and k from 2 reference frames U and V respectively. Distance between refrence frames is L

Hence , d= L+k

Transport theorem
; Let N and B be two frames with a

relative angular velocity vector of mB/N' and let r be a generic vector; then

the derivative of r in the N frame can be related to the derivative of r in

the B frame as

Nd Bd

dt ( r) = dt ( r) + m BIN x r

velocity vector of particle with respect to u and v

v

velocity vector of origin of v with respect to u

by transport theorem velocity vector of particle with respect to A

hence acceleration of particle with respect to A

# Domain A – Vector Kinematics

(Based on Chapter 1: Particle Position Description, Angular Velocity, Moving Frames, and Transport Theorem)

## 1. Reference Frames and Vector Representation

Describing the motion of a particle in space requires a reference frame.
A reference frame is fully defined by:

- An origin

- Three mutually perpendicular unit direction vectors

If a particle is located at a distance d from the origin of a reference frame RR, its position vector can be expressed as a 3×1 column matrix built from the components along the unit vectors of that frame.

### Key idea

Different coordinate systems → different component representations.
Before performing vector operations, all vectors must be projected into the same reference frame.

## 2. Coordinate Systems

### 2.1 Cartesian Coordinates

- Uses fixed perpendicular axes (x,y,z)(x,y,z).

- Position is written as r=x i^+y j^+z k^r=xi^+yj^​+zk^.

- Most convenient for linear motions or problems with fixed orientations.

The Cartesian basis does not rotate, so its unit vectors are constant.

### 2.2 Cylindrical Coordinates

Useful when a particle undergoes rotational motion, or the dominant force acts radially.
The coordinate set is:

- rr: radial distance

- θθ: angular position

- zz: position along the axis of rotation

Two basis vectors lie in the plane perpendicular to the axis and rotate with angle θθ; the third is fixed along the axis.
Often, one of the three entries remains zero for planar rotation.

### Position Vector in Cylindrical Coordinates

r=r e^r+z e^zr=re^r​+ze^z​

But in full vector form:

r=rcos⁡θ i^+rsin⁡θ j^+z k^r=rcosθi^+rsinθj^​+zk^

### Unit Vectors

The cylindrical basis rotates with angle θθ:

e^r=cos⁡θ i^+sin⁡θ j^e^r​=cosθi^+sinθj^​e^θ=−sin⁡θ i^+cos⁡θ j^e^θ​=−sinθi^+cosθj^​e^z=k^e^z​=k^

### Velocity in Cylindrical Coordinates

v=r˙ e^r+rθ˙ e^θ+z˙ e^zv=r˙e^r​+rθ˙e^θ​+z˙e^z​

### Acceleration in Cylindrical Coordinates

a=(r¨−rθ˙2) e^r+(rθ¨+2r˙θ˙) e^θ+z¨ e^za=(r¨−rθ˙2)e^r​+(rθ¨+2r˙θ˙)e^θ​+z¨e^z​

### 2.3 Spherical Coordinates

Used when a particle moves:

- on the surface of a sphere

- or orbits a central body

The orientation is described by two angles:

- θθ: polar angle

- ϕϕ: azimuthal angle

Its basis vectors continuously rotate as the particle moves on the spherical surface.

### Position Vector in Spherical Coordinates

r=ρ e^ρr=ρe^ρ​

Expanded in Cartesian components:

r=ρsin⁡θcos⁡ϕ i^+ρsin⁡θsin⁡ϕ j^+ρcos⁡θ k^r=ρsinθcosϕi^+ρsinθsinϕj^​+ρcosθk^

### Unit Vectors

e^ρ=sin⁡θcos⁡ϕ i^+sin⁡θsin⁡ϕ j^+cos⁡θ k^e^ρ​=sinθcosϕi^+sinθsinϕj^​+cosθk^e^θ=cos⁡θcos⁡ϕ i^+cos⁡θsin⁡ϕ j^−sin⁡θ k^e^θ​=cosθcosϕi^+cosθsinϕj^​−sinθk^e^ϕ=−sin⁡ϕ i^+cos⁡ϕ j^e^ϕ​=−sinϕi^+cosϕj^​

### Velocity in Spherical Coordinates

v=ρ˙ e^ρ+ρθ˙ e^θ+ρsin⁡θ ϕ˙ e^ϕv=ρ˙​e^ρ​+ρθ˙e^θ​+ρsinθϕ˙​e^ϕ​

### Acceleration in Spherical Coordinates

a=  (ρ¨−ρθ˙2−ρsin⁡2θ ϕ˙2)e^ρ+(ρθ¨+2ρ˙θ˙−ρsin⁡θcos⁡θ ϕ˙2)e^θ+(ρsin⁡θ ϕ¨+2ρ˙sin⁡θ ϕ˙+2ρcos⁡θ θ˙ϕ˙)e^ϕa=​(ρ¨​−ρθ˙2−ρsin2θϕ˙​2)e^ρ​+(ρθ¨+2ρ˙​θ˙−ρsinθcosθϕ˙​2)e^θ​+(ρsinθϕ¨​+2ρ˙​sinθϕ˙​+2ρcosθθ˙ϕ˙​)e^ϕ​​

These are the full expressions used in rigid-body, orbital, and spacecraft kinematics.

## 3. Angular Velocity Vector

The angular velocity vector describes how fast the position vector (radius vector) of a particle sweeps around the origin.

If a body rotates about a fixed axis by a small angle dθdθ, its angular velocity is:

ω=θ˙ e^axisω=θ˙e^axis​

The direction of ωω follows the right-hand rule.

Angular velocity exists even if the particle’s magnitude of radius is constant—it measures the rate of rotation of the radius vector.

## 4. Kinematics in Moving Reference Frames

Suppose a particle’s position is measured from two reference frames:

- Frame UU: origin at OUOU​, and particle distance = dd

- Frame VV: origin at OVOV​, distance from particle = kk

- Distance between the two origins = LL

Then,

d=L+kd=L+k

This relation allows us to express position in either frame.

## 5. The Transport Theorem (Time Derivative in Moving Frames)

Let:

- NN = inertial (or reference) frame

- BB = moving frame

- ωB/NωB/N​ = angular velocity of frame BB relative to frame NN

- rr = any vector expressed in frame BB

The transport theorem states:

(drdt)N=(drdt)B+ωB/N×r(dtdr​)N​=(dtdr​)B​+ωB/N​×r

Interpretation:

- The derivative in the inertial frame =
derivative seen in the rotating frame plus a correction due to the frame’s rotation.

- This correction term generates Coriolis, centripetal, and transverse accelerations when applied to velocity.

This is the most fundamental tool for analyzing motion in spacecraft, aircraft, and rotating coordinate systems.

## 6. Velocity Relations Between Frames

If a particle has velocity in frame VV, and frame VV itself moves relative to frame UU, then:

vP/U=vOV/U+(drdt)V+ωV/U×rvP/U​=vOV​/U​+(dtdr​)V​+ωV/U​×r

Where:

- vP/UvP/U​: velocity of particle w.r.t. frame UU

- vOV/UvOV​/U​: velocity of origin of frame VV w.r.t. UU

- ωV/UωV/U​: angular velocity of frame VV relative to UU

This expression directly comes from the transport theorem.

## 7. Acceleration in a Moving Frame

Applying the transport theorem again for velocity gives the full acceleration expansion:

aP/U=aOV/U+(dvdt)V+2ωV/U×vrel+ωV/U×(ωV/U×r)+ω˙V/U×raP/U​=aOV​/U​+(dtdv​)V​+2ωV/U​×vrel​+ωV/U​×(ωV/U​×r)+ω˙V/U​×r

Contains:

- Relative acceleration

- Coriolis acceleration 2ω×vrel2ω×vrel​

- Centripetal (radial) acceleration ω×(ω×r)ω×(ω×r)

- Transverse acceleration ω˙×rω˙×r

This is the complete kinematic description used in rigid-body and spacecraft dynamics.