# A.4.1 Six-DOF picture and the “rotational-only” reduction

A rigid body has 6 DOF: translation of the center of mass (COM) + rotation about the COM.

We often decouple them in attitude dynamics by choosing the reference point at the COM (or using an inertial reference point), so the rotational dynamics can be treated via angular momentum balance rather than Newton’s second law on a point mass.

Key shift: rotation is governed by the moment balance

MO=dHOdt∣NMO​=dtdHO​​​N​

not by F=maF=ma directly.

# A.4.2 Angular momentum: orbital + spin decomposition

For mass elements dmdm, angular momentum about a point OO is

HO=∫r×R˙ dmHO​=∫r×R˙dm

(rr: measured from OO; RR: inertial position).

Decompose motion into COM motion + relative motion about COM:

- Orbital part (depends on COM translation)

- Spin part (pure rotation about COM)

When OO is the COM (common in spacecraft attitude work), HH is dominated by the spin part and the rotational balance simplifies cleanly.

Physical meaning: torque doesn’t “create” angular momentum from nothing — it changes the body’s existing angular momentum vector (magnitude and/or direction).

📌 Figure insertion: body/inertial frames, COM, vectors rr, RR, RcRc​.

# A.4.3 Body frame, transport theorem, and why dynamics become nonlinear

Introduce a body-fixed frame B={b1,b2,b3}B={b1​,b2​,b3​} embedded in the rigid body (typically aligned with principal axes later).

Any vector vv satisfies the transport theorem:

(dvdt)N=(dvdt)B+ω×v(dtdv​)N​=(dtdv​)B​+ω×v

This single identity is why even a “simple rigid body” produces coupled nonlinear ODEs: the frame itself rotates.

# A.4.4 Inertia matrix / dyadic and the inertia geometry

In the body frame, define inertia (about the chosen point, typically COM):

H=J ωH=Jω

with JJ symmetric positive definite.

Dyadic form (coordinate-free):

J^=∫(ρ2I^−ρρT) dmJ=∫(ρ2I−ρρT)dm

(ρρ: position from the reference point).

Why it matters: JJ is the “shape-of-mass” operator that turns angular velocity into angular momentum; it is the object that creates coupling (products of inertia) and sets stability properties.

📌 Figure insertion: inertia dyadic geometry / mass distribution picture.

# A.4.5 Parallel-axis theorem (offset reference points)

If you know inertia about the COM, inertia about another point OO offset by rcrc​ is

J^O=J^C+m(rc2I^−rcrcT)JO​=JC​+m(rc2​I−rc​rcT​)

This becomes unavoidable for:

- actuator lines not passing through COM,

- appendages / booms,

- internal moving masses, fuel slosh approximations.

# A.4.6 Principal axes and diagonalisation

There exists a special orthonormal body frame (principal axes) such that

J=diag(J1,J2,J3),J1≥J2≥J3 (common ordering)J=diag(J1​,J2​,J3​),J1​≥J2​≥J3​ (common ordering)

found from the eigenvalue problem

(λI−J)e=0.(λI−J)e=0.

Interpretation: in principal axes, the inertia operator is “as decoupled as it can be,” and Euler’s equations become clearest.

📌 Figure insertion: principal axes vs an arbitrary body frame.

# A.4.7 Euler’s rotational equations (vector and component forms)

Angular momentum balance in the inertial frame:

M=dHdt∣NM=dtdH​​N​

Convert to body frame using the transport theorem:

M=dHdt∣B+ω×HM=dtdH​​B​+ω×H

Substitute H=JωH=Jω (constant JJ in the body frame for a rigid body about a fixed point):

M=Jω˙+ω×(Jω)  M=Jω˙+ω×(Jω) ​

In principal axes:

J1ω˙1−(J2−J3)ω2ω3=M1J2ω˙2−(J3−J1)ω3ω1=M2J3ω˙3−(J1−J2)ω1ω2=M3J1​ω˙1​−(J2​−J3​)ω2​ω3​J2​ω˙2​−(J3​−J1​)ω3​ω1​J3​ω˙3​−(J1​−J2​)ω1​ω2​​=M1​=M2​=M3​​

Nonlinear + coupled: even constant torque can lead to rich behaviour.

# A.4.8 Torque-free motion: two invariants and geometric picture

For M=0M=0:

- Angular momentum magnitude is conserved

- Rotational kinetic energy is conserved

T=12ωTJω=const,H=Jω,  ∣H∣=constT=21​ωTJω=const,H=Jω,  ∣H∣=const

These invariants constrain ωω to move along the intersection of two surfaces in ωω-space:

- energy ellipsoid

- angular-momentum ellipsoid

The trajectory of ωω in the body frame is the polhode; the corresponding curve traced in inertial space is the herpolhode.

📌 Figure insertion: polhode/herpolhode geometry.

# A.4.9 Stability of torque-free spin about principal axes (tennis-racket effect)

For a rigid body with J1>J2>J3J1​>J2​>J3​, pure spin about a principal axis is an equilibrium of the torque-free system. Linear stability results:

- spin about maximum inertia axis (J1J1​) → stable (Lyapunov)

- spin about minimum inertia axis (J3J3​) → stable (Lyapunov)

- spin about the intermediate axis (J2J2​) → unstable

This is the classical intermediate-axis instability (often visualised as the “tennis racket / Dzhanibekov” phenomenon).

📌 Figure insertion: stability interpretation via polhode projection (elliptic vs hyperbolic character).

# A.4.10 Nondimensional torque-free Euler system and its integrals (from your Problem 6.7 page)

A useful scaled form (especially for theory and phase-space analysis) rewrites the torque-free dynamics into a symmetric-looking set:

x˙1−x2x3=0,x˙2+x3x1=0,x˙3−x1x2=0x˙1​−x2​x3​=0,x˙2​+x3​x1​=0,x˙3​−x1​x2​=0

(derivatives with respect to a scaled time ττ, and xixi​ scaled from ωiωi​ using inertia-dependent constants.)

## (i) Two constants of motion (integrals)

Multiply and add strategically:

- Differentiate x22+x32x22​+x32​:

ddτ(x22+x32)=2x2x˙2+2x3x˙3=2x2(−x3x1)+2x3(x1x2)=0dτd​(x22​+x32​)=2x2​x˙2​+2x3​x˙3​=2x2​(−x3​x1​)+2x3​(x1​x2​)=0

So

x22+x32=Ax22​+x32​=A​

- Differentiate x12+x22x12​+x22​:

ddτ(x12+x22)=2x1x˙1+2x2x˙2=2x1(x2x3)+2x2(−x3x1)=0dτd​(x12​+x22​)=2x1​x˙1​+2x2​x˙2​=2x1​(x2​x3​)+2x2​(−x3​x1​)=0

So

x12+x22=Bx12​+x22​=B​

These imply handy identities (often used to eliminate variables):

x12−x32=B−A,x12+2x22+x32=A+B.x12​−x32​=B−A,x12​+2x22​+x32​=A+B.

## (ii) Second-order form + “decoupled” nonlinear oscillators

Differentiate the first equation x˙1=x2x3x˙1​=x2​x3​:

x¨1=x˙2x3+x2x˙3=(−x3x1)x3+x2(x1x2)=−(x32−x22)x1x¨1​=x˙2​x3​+x2​x˙3​=(−x3​x1​)x3​+x2​(x1​x2​)=−(x32​−x22​)x1​

so

x¨1+(x32−x22)x1=0x¨1​+(x32​−x22​)x1​=0​

Similarly,

x¨2+(x12+x32)x2=0,x¨3+(x12−x22)x3=0.x¨2​+(x12​+x32​)x2​=0​,x¨3​+(x12​−x22​)x3​=0​.

Now substitute the invariants:

x32−x22=A−2x22=A−2(B−x12)=A−2B+2x12x32​−x22​=A−2x22​=A−2(B−x12​)=A−2B+2x12​

giving the cubic nonlinearity form:

x¨1+(A−2B)x1+2x13=0x¨1​+(A−2B)x1​+2x13​=0​

Likewise,

x¨2+(A+B)x2−2x23=0x¨2​+(A+B)x2​−2x23​=0​x¨3+(B−2A)x3+2x33=0x¨3​+(B−2A)x3​+2x33​=0​

These are “decoupled” in the sense that each equation is written in a single variable, but they remain nonlinear (Duffing-type).

Insight: the torque-free rigid body hides a structured nonlinear oscillator system once you use invariants correctly — this is why phase portraits and separatrices show up naturally.

# A.4.11 Constant body-fixed torque: equilibria, manifolds, separatrices (your Fig. 6.5–6.7 style pages)

When a constant torque is applied along a body axis, Euler’s equations become forced. After nondimensionalization (common in attitude texts), one sees forms like:

- one equation picks up a constant “+1” forcing,

- the others remain coupled through products.

The key qualitative consequences:

### (i) Equilibrium points and linear stability types

Setting time derivatives to zero yields equilibrium sets (isolated points or curves depending on which torque component is active). Linearization around these equilibria produces characteristic polynomials whose roots classify them as stable/unstable.

A common outcome (as in stability tables) is: some equilibria are stable only within parameter inequalities involving inertia ratios and equilibrium spin components.

### (ii) Stable/unstable regions and the role of separatrices

In the reduced phase plane (often (x,θ)(x,θ) or projections like (x2,x3)(x2​,x3​)):

- separatrices form boundaries dividing qualitatively different motion classes,

- trajectories on one side remain bounded (libration-like),

- on the other side they “escape” (unbounded growth in the reduced coordinate, or transition to another rotational regime).

📌 Figure insertion:

- stability diagram in the (x2,x3)(x2​,x3​) plane with stable vs unstable arcs (your circular boundary picture),

- separatrix surfaces in (x1,x2,x3)(x1​,x2​,x3​) space (the 3D mesh surfaces labeled stable/unstable regions).

### (iii) Engineering meaning

This is the mathematical language of:

- continuous-thrust misalignment causing coning/precession,

- “spin-up” control where torque pushes the system across a separatrix,

- sensitive dependence near boundaries (small initial condition changes → different long-term rotation regimes).

# A.4.12 Spinning axisymmetric body with a constant transverse torque (thrust misalignment model)

For an axisymmetric body J1=J2=JJ1​=J2​=J, with spin ω3≈nω3​≈n approximately constant, constant misalignment torque generates coupled transverse dynamics often written (after simplification) as:

ω˙1=λω2+μ,ω˙2=−λω1ω˙1​=λω2​+μ,ω˙2​=−λω1​

where λλ depends on (J−J3)(J−J3​) and the spin rate nn, and μμ is proportional to the applied transverse torque level.

These yield sinusoidal solutions for ω1,ω2ω1​,ω2​ with an offset driven by μμ, and (with small-angle kinematics) produce combined precession + nutation of the symmetry axis. The trajectory of the tip of the symmetry axis in a transverse plane can become an epicycloid-like curve depending on parameter ratios.

📌 Figure insertion: the page with ω1,ω2ω1​,ω2​ linear system and the derived precession/nutation amplitudes and frequencies.

# A.4.13 Orbit–attitude coupling: gravity-gradient torque and LVLH dynamics (your Fig. 6.8 and Eq. cluster around 6.142–6.153)

For a rigid body in a circular orbit, gravity is slightly stronger at the near side than the far side. The net force passes through the COM (so no net translation torque), but the moment distribution produces a gravity-gradient torque.

A compact and widely used vector/dyadic expression is:

Mgg=3n2 a3×(Ja3)  Mgg​=3n2a3​×(Ja3​) ​

where:

- nn is orbital rate,

- a3a3​ is the LVLH radial direction (toward Earth), expressed in the body frame when inserted into the cross product.

This torque couples orbital geometry to attitude: it tries to align the axis with minimum energy orientation relative to LVLH.

## Body rate relationship in orbit (LVLH vs inertial)

If the LVLH frame rotates at nn relative to inertial, the spacecraft angular velocity relative inertial includes the relative rotation plus the LVLH rotation contribution. This is why the kinematic mapping includes nn-dependent terms, and why the “same” rigid body behaves differently in orbit than in deep space.

📌 Figure insertion: rigid body in circular orbit showing LVLH axes, body axes, position vectors.

# A.4.14 Linearised attitude equations in LVLH: stability conditions (roll/pitch/yaw coupling)

For small attitude angles relative to LVLH, the dynamics linearize into a set of coupled second-order equations for θ1,θ2,θ3θ1​,θ2​,θ3​ (roll/pitch/yaw-like angles relative to LVLH) with coefficients depending on inertia differences and nn.

A typical structure emerges:

- Pitch often decouples and yields a scalar characteristic equation; the sign condition involves inertia ordering (e.g., a condition like J1>J3J1​>J3​ appears as a stability requirement in certain configurations).

- Roll/yaw remain coupled and produce a higher-order characteristic polynomial with stability conditions expressed through inertia ratios such as:

k1=J2−J3J1,k3=J2−J1J3,k1​=J1​J2​−J3​​,k3​=J3​J2​−J1​​,

and inequalities that ensure all characteristic roots are imaginary (Lyapunov stability) rather than having positive real parts (instability).

Engineering meaning: gravity-gradient “passive stability” is real, but it is directional: some axes stabilize, others can destabilize depending on the inertia ordering and the attitude configuration.

📌 Figure insertion: the pages showing the decoupled pitch characteristic equation and the roll/yaw characteristic polynomial and inequalities.

# A.4.15 Gyrostats and internal angular momentum (reaction wheels / momentum bias)

A gyrostat is a rigid body carrying internal spinning rotors (reaction wheels or a momentum wheel). Total angular momentum is:

H=Jω+hH=Jω+h

where hh is the wheel/rotor angular momentum expressed in the body frame.

Dynamics become:

M=ddt∣B(Jω+h)+ω×(Jω+h)M=dtd​​B​(Jω+h)+ω×(Jω+h)

Even with small angles in orbit, hh introduces bias stiffness and cross-coupling terms that strongly affect stability (classic momentum-bias spacecraft behavior).

📌 Figure insertion: “gyrostat in circular orbit” diagram showing wheel axes and LVLH reference.

# A.4.16 Dual-spin spacecraft with a platform damper: internal moving mass + dissipation

Dual-spin designs (spinning rotor + despun platform) introduce:

- multiple rotating sub-bodies,

- internal relative rates,

- and often damping devices to remove nutation energy.

A simplified but powerful model includes:

- main body inertia about a reference point OO,

- an internal moving/damper mass connected by spring-damper elements,

- an external rotor with its own spin rate and control torque.

You typically see:

- a rotational momentum balance for the composite, including terms like rc×a0rc​×a0​ when the composite COM is offset and accelerating,

- rotor equation (spin dynamics),

- damper mass equation along a body axis with stiffness kk and damping cc, plus coupling terms involving ωiωi​.

What this teaches (big picture):

- internal dissipation shrinks the “energy surface,”

- the system migrates toward stable spin states,

- the final spin axis selection depends on inertia ordering and damping distribution (why some dual-spin vehicles settle reliably while others tumble without damping).

📌 Figure insertion: dual-spinner + despun platform damper schematic and the coupled equations page (with spring/damper + rotor).

# A.4.17 Why Domain A.4 matters in real GNC

- Control torque → angular acceleration is not aligned with torque unless inertia is spherical.

- Coupling is the norm: a commanded M1M1​ can create motion in ω2,ω3ω2​,ω3​ through ω×(Jω)ω×(Jω).

- Stability is not “intuition,” it’s math: intermediate-axis instability, separatrices, orbit-coupled modes, and gyrostatic stiffening are all dynamic effects.

- In orbit, attitude dynamics are never purely “rigid-body in free space”: LVLH rotation and gravity gradient reshape the equations.

Kinematics tells you how orientation changes once rotation is known.
Dynamics tells you what rotation actually happens — and whether it is stable.

| Section | Best source |
| --- | --- |
| Six-DOF picture | Your own redraw |
| Angular momentum decomposition | Your redraw |
| Transport theorem | Your redraw |
| Inertia dyadic geometry | Hughes / redraw |
| Parallel-axis geometry | Your redraw |
| Principal axes | Schaub & Junkins / redraw |
| Euler equations | No figure needed |
| Polhode / herpolhode | Schaub & Junkins → redraw later |
| Intermediate-axis instability | Schaub & Junkins |
| Nondimensional phase portraits | Your own MATLAB / Python |
| Constant torque separatrices | Your Problem 6.7 figures (redraw) |
| Axisymmetric spinner | Wie |
| Gravity gradient | Wie / NASA |
| LVLH linearised stability | Wie |
| Gyrostat | Hughes |
| Dual-spin + damper | Schaub & Junkins |